Product: SIYI A8 mini camera

** Refer to the User Manual found in this page**

System Overview for Developers

The entire system runs on a Raspberry Pi 4B. All the code, configuration files, and services live on the Pi. You access and edit everything by SSHing into the Pi from your laptop. There is no code running on the laptop - the laptop is just a browser and VLC viewer.

Accessing the Pi

Connect your laptop to the AstaPi Wi-Fi hotspot (password: lakrits1), then open a terminal and run:

ssh astapi@192.168.4.1

or if that doesn’t work

ssh astapi@AstaPi.local

Enter the password (lakrits). You are now inside the Pi and can run any command, edit any file, and manage any service. If you are at home and the Pi is connected to your home network, you can also SSH in using:

ssh astapi@10.3.11.78 (I think)

File Locations

Everything relevant to the system lives in  /home/astapi/. The most important files are:

- /home/astapi/gimbal_control.py - Flask web application and REST API; gimbal control logic; edit this to add new endpoints, change existing behavior, or modify the web interface

- /home/astapi/mediamtx.yml - MediaMTX configuration file. Edit this to change streaming settings, ports, or stream sources

- /home/astapi/mediamtx - MediaMTX executable; do not modify this file

- /home/astapi/siyi_sdk/ - SIYI SDK folder;  contains the Python library that talks to the camera. You generally do not need to modify this unless updating the SDK

- /etc/systemd/system/gimbal.service -  the service file that auto-starts the Flask app on boot

- /etc/systemd/system/mediamtx.service - the service file that auto-starts MediaMTX on boot

Editing Files

To edit any file on the Pi, use the nano text editor. For example to edit the Flask app:

nano ~/gimbal_control.py

Save with Ctrl+X, then Y, then Enter. Changes do not take effect until you restart the relevant service.

Restarting Services After Changes

After editing gimbal_control.py, restart the gimbal service:

sudo systemctl restart gimbal

After editing mediamtx.yml, restart the MediaMTX service:

sudo systemctl restart mediamtx

To check if a service is running correctly after a restart:

sudo systemctl status gimbal

sudo systemctl status mediamtx

Look for active (running) in green. If it shows failed, there is a syntax error or crash in the code. View the error with:

sudo journalctl -u gimbal -n 30

Modifying the Web Interface

The web interface HTML lives inside the index() function in gimbal_control.py as a large string. It is standard HTML, CSS, and JavaScript. To modify the appearance or add new buttons, edit the HTML string directly. For example to add a button that calls a new endpoint, add this inside the HTML:

<button onclick="send('/surveillance_position')">Surveillance</button>

Save and restart the gimbal service to see the changes.

Changing the Wi-Fi Hotspot Password or Name

To change the hotspot SSID or password:

sudo nmcli con modify hotspot ssid "NewNetworkName"

sudo nmcli con modify hotspot wifi-sec.psk "NewPassword"

sudo nmcli con up hotspot

Changing the Camera IP

If the camera's IP address ever changes, update it in two places. First in gimbal_control.py, find this line near the top and update the IP:

cam = SIYISDK(server_ip="192.168.144.25", port=37260)

Second in mediamtx.yml, find this line and update the IP:

source: rtsp://192.168.144.25:8554/main.264

Restart both services after making these changes.

Changing Ports

The Flask app runs on port 5000 by default. To change it, find this line at the bottom of gimbal_control.py:

app.run(host='0.0.0.0', port=5000)

Change 5000 to any available port. MediaMTX ports are controlled in mediamtx.yml — the RTSP port is set by rtspAddress: :8554 and the HLS port by hlsAddress: :8888.

Disabling Auto-start

If you want a service to stop starting automatically on boot:

sudo systemctl disable gimbal

sudo systemctl disable mediamtx

To re-enable:

sudo systemctl enable gimbal

sudo systemctl enable mediamtx

Getting Started - Gimbal Camera System

What you need:

- A laptop or phone with Wi-Fi and a browser

- VLC Media Player installed

- The drone powered on

Step 1 - Power on the drone

Turn on the drone and wait approximately 45 seconds. The Pi boots automatically, starts all services, and begins broadcasting the Wi-Fi hotspot. You do not need to do anything on the Pi itself.

Step 2 - Connect to the Wi-Fi hotspot

On your laptop network named AstaPi. Enter the password (lakrits1) when prompted. Note that once connected, your device will not have internet access - AstaPi is a private network with no internet connection.

Step 3 - Access the gimbal control interface

Open any web browser (Chrome, Edge, Safari, Firefox) and type the following address into the address bar exactly as written:

http://192.168.4.1:5000

Make sure to include http:// - do not let the browser add https:// automatically. The gimbal control panel will load. From here you can:

- Press and hold the Up, Down, Left, Right buttons to pan and tilt the camera

- Use the speed slider to adjust how fast the gimbal moves

- Use the zoom slider to zoom in and out (1x to 6x)

- Enter specific yaw and pitch values in the Absolute Angle fields and press Go to move to an exact position

- Press Center to return the gimbal to its default forward-facing position

- Press Patrol Scan, Horizon Sweep, or Nod Search to run automated scanning patterns

- Press Photo to take a still image saved to the camera's SD card

- Press Record to start video recording, and Stop Rec to stop

The angle display at the top of the page shows the gimbal's current yaw, pitch, and roll in real time, updating every second.

If controls aren’t responsive:
try running: python3 gimbal_control.py

If no response, enter: CTRL + C. Then type: sudo systemctl stop gimbal. Then run  python3 gimbal_control.py again

Step 4 - View the live video feed

Open VLC Media Player on your laptop. Go to Media, then Open Network Stream, and enter the following address:

rtsp://192.168.4.1:8554/camera

Click Play. The live video feed from the camera will appear within a few seconds. If it does not appear, wait a moment and try again - the stream may still be initializing.

Alternatively, if you want to view the stream in a browser, navigate to:

http://192.168.4.1:8888/camera/index.m3u8

Troubleshooting

If the control page doesn't load, confirm your device is connected to AstaPi Wi-Fi and not your home network. Open a Command Prompt or Terminal and run ping 192.168.4.1 - if you get replies, the Pi is reachable. If not, wait another 30 seconds for the Pi to finish booting and try again.

If the video stream doesn't play in VLC, the camera Ethernet cable may have come loose. The cable connects the camera to the Pi and can be sensitive to vibration. Reseat it firmly on both ends and wait a few seconds for the stream to reconnect.

If the gimbal controls appear to work (the page responds) but the camera doesn't physically move, the camera may have lost power or the Ethernet cable is disconnected. Check both connections.

If you need to access the Pi directly for any reason, open a terminal and SSH in using:

ssh astapi@192.168.4.1

The password is the Pi's user password set during initial setup (lakrits)